Project GR001 IP AXI_SRAMC

Hardware Design Document

Provided by   Neo Wang                      Provided Date

Approved by                                  Approved Date

Revision History

| Version | Release Date | Owner | Description |
| --- | --- | --- | --- |
| V0.1 | 2023/8/7 | Neo Wang | Initial Version |
| V0.5 | 2023/9/11 | Neo Wang | Updated uarch & split ecc |
| V0.9 | 2023/9/22 | Neo Wang | Updated parameter options |

Attention: The blue italics are template comments, please remove all of them before releasing the document！

注意： 蓝色斜体字部分是模版注释内容，请在正式发布的版本中将他们全部移除！

Contents

Revision History	2

Contents	4

Table List	6

Figure List	7

1	Introduction	8

2	Feature List	9

3	Functional Description	10

3.1	Architecture	10

3.2	Design Description	10

3.3	High Level Timing Diagram	11

3.4	Clock and Reset	11

3.4.1	Clock	11

3.4.2	Reset	11

3.5	I/O Interfaces	11

3.6	Address Mapping	13

4	Block Description	14

4.1	Read Handler and Write Handler	14

4.2	Arbiter and Decoder	16

4.3	Memory interface control	17

5	Use cases	18

5.1	Scenario 1	18

5.2	Scenario 2	19

6	Configuration	20

6.1	Parameter	20

6.2	Macro	20

6.3	Memory	20

7	PPA Information	21

7.1	Power	21

7.2	Performance	21

7.3	Area	21

8	Implementation Constraints	23

9	P & R Guidelines	24

10	Verification	25

11	Appendix	26

11.1	ECC Wrapper	26

11.2	Memory Wrapper (TBD)	26

Table List

Table 3-1 Clock Requirements	11

Table 3-2 Reset Requirements	11

Table 3-3 I/O Interfaces (see 6 Configuration for configurable width)	13

Table 3-4 Address Mapping	13

Table 4-1 Request/Response Format of Arbiter/Decoder	15

Table 6-1 Parameter List	20

Table 6-2 Macro List	20

Table 6-3 Memory Requirements Table	20

Table 7-1 Power State Table	21

Table 7-2 Power Consumption Table	21

Table 7-3 Area Information	21

Table 7-4 VT Cell Number	22

Figure List

Figure 3-1 AXI_SRAMC architecture Diagram	10

Figure 3-2 High Level Timing Diagram	11

Figure 4-1 Read Handler Diagram	14

Figure 4-2 Write Handler Diagram	15

Figure 4-3 RMW State Machine Diagram	16

Figure 4-4 Mem_if_ctrl Diagram	17

Figure 5-1 Data Path of Normal Write	18

Figure 5-2 Data Path of Partial Write	19

Figure 11-1 ECC Wrapper Diagram	26

Figure 11-2 Memory Wrapper Diagram	26

Introduction

Please provide brief and high-level description here for the IP or SYS. For example, design background and basic IP functions.

AXI_SRAMC is a SRAM memory controller based on the standard AXI bus. It provides an AXI4 slave interface that can be connected to AXI4 master or interconnect devices in the AXI4 systems and a user-defined master interface of memory wrapper which based on valid/ready handshake signal.

Feature List

Please list all main features of IP or Sub System here.

FL-001: the AXI4 slave interface supports 32-bit address bus and 64/128/256-bit data bus (the data width corresponds to the SRAM data width)

FL-002: burst transfer supports up to 16 beats for INCR or WRAP burst type

FL-003: supports narrow transfers and unaligned transfers

FL-004: supports RMW operation for ECC

FL-005: configurable Regs Slice

FL-006: only support single-port SRAM

FL-007: supports SRAM capacity: 1KB~8MB

FL-008: supports up to 16 GB/s write or read bandwidth @500MHz

Functional Description

Please provide detailed function description and architecture diagram in this chapter.

Architecture

Figure 3-1 AXI_SRAMC architecture Diagram

Design Description

AXI_SRAMC is responsible for converting AXI access transactions into SRAM access transactions. The upstream AXI master initiates a read or write request to SRAM. AXI transaction is split and converted into a mem request through read/write handler module. And then, it is sent to the downstream Memory Wrapper (See Appendix Section 11.2 for relevant information on Memory Wrapper). In order to complete data error correction and improve robustness, an ECC module (See Appendix Section 11.1 for relevant information on ECC Wrapper) is used to encode written data and decode read data. Because data granularity of the ECC module encoding and decoding is the bus bit width, for AXI write transaction of partial write, Module Write Handler needs to initiate a read operation to obtain complete write data before continuing the write operation.

High Level Timing Diagram

Please provide the high level I/O timing diagram for the whole IP or SYS.

Figure 3-2 High Level Timing Diagram

As shown in Figure 3-2, there are three AXI write operations, the first is a normal write transaction (NW) with “wstrb==`1” and “burst length==3”, the second is a partial write transaction (PW) with “wstrb!=`1” and “burst length==2”, the third is a narrow write transacton (NRW) with “burst length==1”.

Clock and Reset

Please provide required clock and reset information in this chapter.

Clock

| Clock Group | Clock Name | Frequency | Duty Cycle | Accuracy |  |
| --- | --- | --- | --- | --- | --- |
| group 1 | clk | 500MHz | 45%~55% |  | ±100 ppm |  |

Table 3-1 Clock Requirements

Reset

| Reset Name | Related Clock | Comments |
| --- | --- | --- |
| rst_n | clk |  |

Table 3-2 Reset Requirements

I/O Interfaces

Please give the fully I/O interfaces in the interface table, the interfaces can be divided into different groups such as:

- Clock

- Reset

- Bus interfaces

- Interrupts

- DFT

- Etcs.

| Category | Interface Name | Width | I/O | Description |
| --- | --- | --- | --- | --- |
| clock | clk | 1 | I | AXI_SRAMC main clock |
| reset | rst_n | 1 | I | Reset signal, active low |
| AXI interfaces | s_awvaild | 1 | I | Write address valid |
|  | s_awready | 1 | O | Write address ready |
|  | s_awid | configurable | I | Write address ID |
|  | s_awaddr | 32 | I | Write address |
|  | s_awlen | 8 | I | AXI burst length |
|  | s_awsize | 3 | I | AXI burst size |
|  | s_awburst | 2 | I | AXI burst type |
|  | s_awcache | 4 | I | Memory type |
|  | s_awprot | 3 | I | Protection type |
|  | s_awqos | 4 | I | Quality of Service |
|  | s_awuser | configurable | I | User signal |
|  | s_wvalid | 1 | I | Write valid |
|  | s_wready | 1 | O | Write ready |
|  | s_wdata | configurable | I | Write data |
|  | s_wstrb | configurable | I | Write strobes |
|  | s_wlast | 1 | I | Write last |
|  | s_bvaild | 1 | O | Write response valid |
|  | s_bready | 1 | I | Response ready |
|  | s_bid | configurable | O | Response ID tag |
|  | s_bresp | 2 | O | Write response |
|  | s_arvaild | 1 | I | Read address valid |
|  | s_arready | 1 | O | Read address ready |
|  | s_arid | configurable | I | Read address ID |
|  | s_araddr | 32 | I | Read address |
|  | s_arlen | 8 | I | AXI burst length |
|  | s_arsize | 3 | I | AXI burst size |
|  | s_arburst | 2 | I | AXI burst type |
|  | s_arcache | 4 | I | Memory type |
|  | s_arprot | 3 | I | Protection type |
|  | s_arqos | 4 | I | Quality of Service |
|  | s_aruser | configurable | I | User signal. |
|  | s_rvaild | 1 | O | Read valid |
|  | s_rready | 1 | I | Read ready |
|  | s_rid | configurable | O | Read ID tag |
|  | s_rdata | configurable | O | Read data |
|  | s_rresp | 2 | O | Read response |
|  | s_rlast | 1 | O | Read last |
| Memory interfaces | m_mi_ce | 1 | O | Memory access valid |
|  | m_mi_we | 1 | O | 0: read; 1: write |
|  | m_mi_addr | 32 | O | Memory access address |
|  | m_mi_data | configurable | O | Memory write data |
|  | m_mo_data | configurable | I | Memory read data |
| Register | axi_sramc_idle | 1 | O | AXI_SRAMC module is idle |
|  | ecc_err | 1 |  | Rdata ECC error and uncorrectable |
|  | ecc_err_addr | configurable |  | Memory address corresponding to the ECC error |

Table 3-3 I/O Interfaces (see 6 Configuration for configurable width)

Address Mapping

| Field Name | Address Range |
| --- | --- |
| SRAM | 0x0~0x80_0000 |

Table 3-4 Address Mapping

Block Description

The IP or SYS could be divided into several sub modules, please provide detailed design information for each sub block in this chapter.

Read Handler and Write Handler

Figure 4-1 Read Handler Diagram

As shown in Figure 4-1, Transfer split is responsible for disassembling the burst transaction and converting it into the request format of the downstream arbiter (see in Table 4-1).

| Name |  |  | Width | Description |
| --- | --- | --- | --- | --- |
| Arbiter req | Vld |  | 1 | Valid signal |
|  | Rdy |  | 1 | Ready signal |
|  | payload | Axid | Configurable | AXI id |
|  |  | Addr | 32 | AXI address |
|  |  | Last | 1 | Last burst transfer flag |
|  |  | User | Configurable | AXI user bit |
|  |  | RW | 1 | 0: read; 1: write |
|  |  | Rmw | 1 | Read request of Write Handler. Reserved for AXI read transaction |
|  |  | Data | Consistent with bus width | Write data. Reserved for AXI read transaction |
| Decoder rsp | Vld |  | 1 | Valid signal |
|  | Rdy |  | 1 | Ready signal |
|  | payload | Axid | Configurable | AXI id |
|  |  | Last | 1 | Last burst transfer response flag |
|  |  | RW | 1 | 0: read; 1: write |
|  |  | Rmw | 1 | Read request of Write Handler. Reserved for AXI read transaction |
|  |  | Data | Consistent with bus width | Read data. Reserved for AXI write transaction |

Table 4-1 Request/Response Format of Arbiter/Decoder

Transfer split supports the burst type split specified in the AXI4 protocol, and the burst length supports a maximum of 16. It is worth noting that the split access address is aligned with the bit width of the SRAM data bus. To simplify the design, it does not support out-of-order reading, that is to say, the Rrsp gen will reply the read data in order, and use the field ‘Last’ in decoder response as the flag bit of the last data. A counter is used to record the read transaction. The counter plus one for the AR receiving a transaction, and counter minus one for the Rrsp gen completing an AR request.

Figure 4-2 Write Handler Diagram

As shown in Figure 4-2, write transaction will also be disassembled and converted in the Transfer split. Module AWW merge is responsible for merging AW & W channel and generating the request packet. Considering the partial write, the write request with ‘wstrb != `1’ will be specially handled in module RMW (Read Modify Write). RMW will first initiate a read operation to read back the data of the complete bus bit width corresponding to the write address, then merge it with the actual data to be written, and finally write the combined data into memory. This process can be abstracted as a state machine in Figure 4-3.

Figure 4-3 RMW State Machine Diagram

For the response returned by memory wrapper, whether it is routed to the RMW or the Brsp gen is determined by field ‘rmw’ in decoder rsp. When ‘rmw’ signal is set, it indicates that the response is a reply to a read request sent by RMW and this response will be routed to RMW. Since out-of-order writing is not supported, the Brsp gen will directly generate write response through field ‘Last’ and ‘Axid’. Similarly, a counter is used to record write transaction and generate a block Write Handler idle signal.

Arbiter and Decoder

As shown in Figure 3-1, a round robin arbiter is employed to arbitrate memory access requests from Write Handler and Read Handler. For the returned response, a decoder is used to complete the shunt. When field rmw==0 && ‘RW’==0, this response will be routed to Read Handler, otherwise routed to write Handler.

  - Memory interface control

Figure 4-4 Mem_if_ctrl Diagram

As shown in Figure 4-4, considering the delay of SRAM output, some control signals for AXI_SRAMC reply and output rdata of SRAM will go through a FIFO to complete the timing alignment. Assuming that the read latency of SRAM is N cycles, the depth of sideband or rdata FIFO is N+1. The output valid is asserted when this two FIFO are not empty(complete response data is available). When sideband FIFO is full, the req ready is unasserted and Memory Wrapper does not accept input access.

Use cases

Please draw all necessary data path flow for typical scenarios, and the necessary description was required.

Scenario 1

Figure 5-1 Data Path of Normal Write

The upstream AXI master initiates a normal write request to SRAM. AXI transaction is split and converted into a memory request in Write Handler. And then, it is sent to ECC Encoder to perform wdata Hamming encoding. Finally, it is sent to the downstream memory wrapper. When mem completes the write operation, it will return the relevant information of the write operation such as id, last flag to Write Handler to generate B channel response.

Scenario 2

Figure 5-2 Data Path of Partial Write

The upstream AXI master initiates a partial write request to SRAM. First, Write Handler splits and converts the transaction, and then Write Handler will first initiate a read request of the corresponding address to memory, and after reading back the data, it will splice the read data with the original AXI partial write data. And finally write the spliced data into memory. For example, AXI original write data is 64’h1111_2222_3333_4444, address is ‘h1234 and wstrb is ‘hfe; Assume that the data stored at address ‘h1234 in memory is 64’h5555_6666_7777_8888 at this time. Then the data finally written to this address ‘h1234 is 1111_2222_3333_4488.

Configuration

Parameter

| Parameter | Default Value | Description |
| --- | --- | --- |
| AXI_ID_WIDTH | 6 | AXI ID width |
| AXI_ADDR_WIDTH | 32 | AXI address width |
| AXI_DATA_WIDTH | 64 | Bus data width, support 64/128/256 |
| AXI_USER_WIDTH | 8 | AXI user width |
| SRAM_R_LATENCY | 1 | Memory read data delay |
| HAS_NARROW_TR | 1 | Whether narrow transfers are supported |
| HAS_UNALIGNED_TR | 1 | Whether unaligned transfers are supported |
| HAS_PARTIAL_W_TR | 1 | Whether partial write transfers are supported |

Table 6-1 Parameter List

Macro

| Macro | Default Value | Description |
| --- | --- | --- |

Table 6-2 Macro List

Memory

| Mem Name | Depth | Width | Frequency | Type | Comments |
| --- | --- | --- | --- | --- | --- |

Table 6-3 Memory Requirements Table

PPA Information

Power

Please give the description regarding the whole low power design methodology in this chapter, and list all of power domain related information in below sheets.

| Power State | Power Domain 1 | Power Domain 2 | Power Domain 3 |
| --- | --- | --- | --- |

Table 7-1 Power State Table

| IP Name | Leakage Power | Internal Power | Switching Power | Total Power |
| --- | --- | --- | --- | --- |

Table 7-2 Power Consumption Table

Performance

Please give the peak performance information in this section.

For example:

- Max Frequency

- Max Bandwidth

- Max Outstanding Number

Please list the performance requirement to achieve the basic function.

- Latency

- Bandwidth

Area

Logic和mem区分开，加个表，包括各个VT cell比例

Please list the area and vt cell information in below tables, the synthesis configurations should be announced.

| Logic Area | Memory Area | Total Area |
| --- | --- | --- |

Table 7-3 Area Information

| SVT Cell Num | LVT Cell Num | ULVT Cell Num | Total Cell Num |
| --- | --- | --- | --- |

Table 7-4 VT Cell Number

Implementation Constraints

Please list all design constraints if have:

For example:

- Multicycle

- False path

- Logical exclusive

- Etcs.

P & R Guidelines

Please provide guidelines for P & R stage.

Verification

Please list special requirements for verification.

Appendix

ECC Wrapper

Figure 11-1 ECC Wrapper Diagram

ECC module is employed to check and correct the memory data, which is shown in Figure 11-1. The ECC module can detect and correct 1-bit errors and detect 2-bit errors.

Memory Wrapper (TBD)

Figure 11-2 Memory Wrapper Diagram