Project GR001 IP AHB_SRAMC

Hardware Design Document

Provided by   Neo Wang                      Provided Date

Approved by                                  Approved Date

Revision History

| Version | Release Date | Owner | Description |
| --- | --- | --- | --- |
| V0.1 | 2023/9/26 | Neo Wang | Initial Version |

Attention: The blue italics are template comments, please remove all of them before releasing the document！

注意： 蓝色斜体字部分是模版注释内容，请在正式发布的版本中将他们全部移除！

Contents

Revision History	2

Contents	3

Table List	5

Figure List	6

1	Introduction	7

2	Feature List	8

3	Functional Description	9

3.1	Architecture	9

3.2	Design Description	9

3.3	High Level Timing Diagram	9

3.4	Clock and Reset	10

3.4.1	Clock	10

3.4.2	Reset	10

3.5	I/O Interfaces	11

3.6	Address Mapping	12

4	Block Description	13

4.1	AHB_SRAMC	13

5	Use cases	14

5.1	Scenario 1	14

5.2	Scenario 2	14

6	Configuration	16

6.1	Parameter	16

6.2	Macro	16

6.3	Memory	16

7	PPA Information	17

7.1	Power	17

7.2	Performance	17

7.3	Area	17

8	Implementation Constraints	19

9	P & R Guidelines	20

10	Verification	21

11	Appendix	22

Table List

Table 3-1 Clock Requirements	10

Table 3-2 Reset Requirements	10

Table 3-3 I/O Interfaces (see 6 Configuration for configurable width)	12

Table 3-4 Address Mapping	12

Table 6-1 Parameter List	16

Table 6-2 Macro List	16

Table 6-3 Memory Requirements Table	16

Table 7-1 Power State Table	17

Table 7-2 Power Consumption Table	17

Table 7-3 Area Information	17

Table 7-4 VT Cell Number	18

Figure List

Figure 3-1 AHB_SRAMC architecture Diagram	9

Figure 3-2 High Level Timing Diagram	10

Figure 5-1 Data Path of Normal Write	14

Figure 5-2 Data Path of Narrow Write	14

Introduction

Please provide brief and high-level description here for the IP or SYS. For example, design background and basic IP functions.

AHB_SRAMC is a SRAM memory controller based on the standard AHB bus. It provides an AHB3 lite slave interface that can be connected to AHB3 master or interconnect devices in the AHB3 Lite systems and a user-defined master interface of memory wrapper.

Feature List

Please list all main features of IP or Sub System here.

FL-001: 32-bit address bus of AMBA3 AHB_Lite slave interface

FL-002: supports 32/64/128/256-bit data bus (the data width corresponds to the SRAM data width)

FL-003: supports bursts of 4, 8, and 16-beats, undefined length bursts, and single transfers

FL-004: supports RMW operation for ECC

FL-005: only support single-port SRAM

FL-006: supports SRAM capacity: 1KB~8M

FL-007: only support same read/write latency for SRAM

FL-008: supports up to 16 GB/s write or read bandwidth @500MHz

Functional Description

Please provide detailed function description and architecture diagram in this chapter.

Architecture

Figure 3-1 AHB_SRAMC architecture Diagram

Design Description

AHB_SRAMC is responsible for converting AHB access transactions into SRAM access transactions. The upstream AHB master initiates a read or write request to SRAM. AHB transaction is converted into a mem request through block mem interface ctrl. And then, it is sent to the downstream Memory Wrapper. In order to complete data error correction and improve robustness, an ECC module is used to encode written data and decode read data. Because data granularity of the ECC module encoding and decoding is the bus bit width, for AHB narrow write transaction, AHB_SRAMC needs to initiate a read operation to obtain complete write data before continuing the write operation.

High Level Timing Diagram

Please provide the high level I/O timing diagram for the whole IP or SYS.

Figure 3-2 High Level Timing Diagram

As shown in Figure 3-2, there are three AHB write operations, the first is a normal write transaction (NW) with “hsize==data width”, the second is a normal read transaction (NR) with “hsize==data width”, the third is a narrow write transacton (NRW) with “hsize!=data width”. Assuming that the latency of SRAM is 1 cycle.

Clock and Reset

Please provide required clock and reset information in this chapter.

Clock

| Clock Group | Clock Name | Frequency | Duty Cycle | Accuracy |  |
| --- | --- | --- | --- | --- | --- |
| group 1 | clk | 500MHz | 45%~55% |  | ±100 ppm |  |

Table 3-1 Clock Requirements

Reset

| Reset Name | Related Clock | Comments |
| --- | --- | --- |
| rstn | clk |  |

Table 3-2 Reset Requirements

I/O Interfaces

Please give the fully I/O interfaces in the interface table, the interfaces can be divided into different groups such as:

- Clock

- Reset

- Bus interfaces

- Interrupts

- DFT

- Etcs.

| Category | Interface Name | Width | I/O | Description |
| --- | --- | --- | --- | --- |
| clock | clk | 1 | I | AHB_SRAMC main clock |
| reset | rstn | 1 | I | Reset signal, active low |
| AHB interfaces | s_hsel | 1 | I | AHB slave select |
|  | s_haddr | 32 | I | AHB access address |
|  | s_hwrite | 1 | I | AHB transfer direction |
|  | s_htrans | 2 | I | AHB transfer type |
|  | s_hsize | 3 | I | AHB transfer size |
|  | s_hburst | 3 | I | AHB transfer burst type |
|  | s_hport | 4 | I | AHB protection control signal |
|  | s_hmastlock | 1 | I | AHB lock sequence |
|  | s_hreadyin | 1 | I | AHB previous transfer complete |
|  | s_hwdata | configurable | I | Write data |
|  | s_hrdata | configurable | O | Read data |
|  | s_hreadyout | 1 | O | Transfer finish signal |
|  | s_hresp | 1 | O | AHB transfer response |
| Memory interfaces | m_mi_ce | 1 | O | Memory access valid |
|  | m_mi_we | 1 | O | 0: read; 1: write |
|  | m_mi_addr | 32 | O | Memory access address |
|  | m_mi_data | configurable | O | Memory write data |
|  | m_mo_data | configurable | I | Memory read data |
| Register | ahb_sramc_idle | 1 | O | AHB_SRAMC module is idle |

Table 3-3 I/O Interfaces (see 6 Configuration for configurable width)

Address Mapping

| Field Name | Address Range |
| --- | --- |
| SRAM | 0x0~0x80_0000 |

Table 3-4 Address Mapping

Block Description

The IP or SYS could be divided into several sub modules, please provide detailed design information for each sub block in this chapter.

AHB_SRAMC

As shown in Figure 3-1 AHB_SRAMC architecture Diagram, this module supports all transfer type and burst type specified in the AHB3 Lite protocol, and the transfer size set by HSIZE must be less than or equal to the width of the data bus. First AHB transfer is converted into a mem request. For write operation, the memory write request can be initiated only after write data has been collected in AHB data phase. For read operations, when the memory returns the read data, the module converts it to the AHB read data and passes it upstream master. It is worth noting that the sram access address is aligned with the bit width of the SRAM data bus.

Considering the narrow write (transfer size less than width of data bus), the write request will be specially handled with RMW (Read Modify Write) operation. RMW operation will first initiate a read operation to read back the data of the complete bus bit width corresponding to the write address, then merge it with the actual data to be written, and finally write the combined data into memory. Since AHB protocol reads and writes are strictly sequence-preserving, SRAMC gains additional operation time by pulling down the ready signal to the upstream master.

Use cases

Please draw all necessary data path flow for typical scenarios, and the necessary description was required.

Scenario 1

Figure 5-1 Data Path of Normal Write

The upstream AHB master initiates a normal read request to SRAM. AHB transaction is converted into a memory request block mem interface ctrl. And then, it is sent to the downstream memory wrapper. When mem completes the read operation, it will return the relevant rdata to AHB_SRAMC to generate B channel response.

Scenario 2

Figure 5-2 Data Path of Narrow Write

The upstream AHB master initiates a narrowwrite request to SRAM. First, block rmw ctrl will initiate a read request of the corresponding address to memory, and after reading back the data, it will splice the read data with the original AHB narrow write data. And finally write the spliced data into memory. For example, AHB original write data is 64’h1111_2222_3333_4444, address is ‘h1234 and hsize is “Word”; Assume that the data stored at address ‘h1230 in memory is 64’h5555_6666_7777_8888 at this time. Then the data finally written to this address ‘h1230 is 1111_2222_7777_8888.

Configuration

Parameter

| Parameter | Default Value | Description |
| --- | --- | --- |
| AHB_DATA_WIDTH | 64 | Bus data width, support 32/64/128/256 |
| SRAM_R_LATENCY | 1 | Memory read data delay |
| HAS_NARROW_TR | 1 | Whether narrow transfers are supported |

Table 6-1 Parameter List

Macro

| Macro | Default Value | Description |
| --- | --- | --- |

Table 6-2 Macro List

Memory

| Mem Name | Depth | Width | Frequency | Type | Comments |
| --- | --- | --- | --- | --- | --- |

Table 6-3 Memory Requirements Table

PPA Information

Power

Please give the description regarding the whole low power design methodology in this chapter, and list all of power domain related information in below sheets.

| Power State | Power Domain 1 | Power Domain 2 | Power Domain 3 |
| --- | --- | --- | --- |

Table 7-1 Power State Table

| IP Name | Leakage Power | Internal Power | Switching Power | Total Power |
| --- | --- | --- | --- | --- |

Table 7-2 Power Consumption Table

Performance

Please give the peak performance information in this section.

For example:

- Max Frequency

- Max Bandwidth

- Max Outstanding Number

Please list the performance requirement to achieve the basic function.

- Latency

- Bandwidth

Area

Logic和mem区分开，加个表，包括各个VT cell比例

Please list the area and vt cell information in below tables, the synthesis configurations should be announced.

| Logic Area | Memory Area | Total Area |
| --- | --- | --- |

Table 7-3 Area Information

| SVT Cell Num | LVT Cell Num | ULVT Cell Num | Total Cell Num |
| --- | --- | --- | --- |

Table 7-4 VT Cell Number

Implementation Constraints

Please list all design constraints if have:

For example:

- Multicycle

- False path

- Logical exclusive

- Etcs.

P & R Guidelines

Please provide guidelines for P & R stage.

Verification

Please list special requirements for verification.

Appendix